BreezeNET PRO.11 - firmware version 5.0.211

This firmware release is valid for the BreezeNET PRO.11 Series, Versions 5.0.x and higher.

Please read the corresponding band release notes for more information about this version's new features, bug fixes, backward compatibility and limitations.

This release notes can be found at: http://www.breezecom.com/TechSupport/techsupp1.html

* General:

1. It is possible to upgrade the BreezeNET units using either the BreezeNET PRO.11 configuration utility, or the TFTP utility that is included in this package.

(For further information on how to use the BreezeNET PRO.11 configuration utility, please refer to the manual at: http://www.breezecom.com/pdfs/pro11_configuration_utility_users_guide.pdf)

2. This document describes only the upgrade procedure using the TFTP DOS based client, which is included in this package.

3. Additional documents referred to on this file, and are not part of this package, may be found on our site: http://www.breezecom.com/TechSupport/techsupp1.html

* Requirements:

1. PC or workstation with an Ethernet NIC and RJ-45 connection, and terminal emulation.

A typical configuration is a PC running Windows with one MS-DOS window and one Hyper Terminal window.

2. Monitor cable (Supplied with AP-10 unit)

3. 10-Base-T cable:

- Regular cable for download of SA-10 or SA-40

- Cross-over cable for download of AP-10 or WB-10

4. The Upgrade kit file: BN_50211.zip.

* Upgrade Procedure:

1. Connect the unit to the power supply.

2. Connect the unit to the terminal emulator using the monitor cable and start a monitor session.

(See your User's Guide for detailed explanation, or refer to:

http://www.breezecom.com/TechSupport/UsersGuide/BreezeNET4_4.htm, chapter 3.)

3. Configure IP address to the unit (See chapter 3 in the User's Guide mentioned above).

* In case you haven't done so in the past, verify that the PC's IP address (from which you are about to upgrade) belongs to the same subnet as the unit.

* If the unit is behind a router, verify that you have set the correct Default GW to the unit.

4. Connect the correct Ethernet cable between the unit and the PC (See chapter 2 in the User's Guide mentioned above).

* NOTE that The Orange LED (Ethernet Link) on the backpanel of the unit should be on at this point.

5. PING the unit's IP address. Make sure that PING replies are being received.

The first one or two PINGs may fail, the rest should be received correctly.

6. Checking the Code (Image) Status: Use the opened monitor session.

From the main menu choose :

5 - Code Activate Control

You will be able to see a screen similar to the below:

Code Activate Control

=====================

1 - Try To Run From NonActive Code

Active Code Version: 5.0.143

NonActive Code Version: 5.0.63 (state unknown)

2 - Check NonActive Code State

*** IMPORTANT! ***

6.1. If the active code version is 5.0.63 or earlier:

You would have to export by TFTP the configuration of each unit that is about to be upgraded.

Use the included TFTP client with the following syntax:

tftp <IP address> get <write community string>.cfg <LocalFileName>.txt

Example :

tftp 206.25.63.55 get private.cfg config.txt

The get TFTP action causes the configuration parameters to be saved on the FLASH memory.

After the upgrade process is completed, the new version: 5.0.211 will restore the saved configuration parameters from the FLASH.

6.2. If the active code version is 5.0.143 to 5.0.211:

Saving of current configuration is done automatically; please continue to the next paragraph.

7. Open a DOS window and use the TFTP client with the following syntax:

* It is recommended to open up the monitor window as well as the dos window in order to see the messages coming up on the screen while the downloading process is performed.

tftp -i hostaddress put sourcefile [destinationfile]

where:

-i  		is for binary mode.

hostaddress 	is the ip address you give the unit.

put		defines that the client will send file to hostaddress.

sourcefile 	is filename to be read from the current directory.

destinationfile 	is filename to be downloaded the unit.

For AP:	sourcefile = N5_0_211.ap, destinationfile = <write community string>

For SA/WB:  sourcefile = N5_0_211.sa, destinationfile = <write community string>

The default write community string is 'private' (omit the quotes).

Example for AP-10:

tftp -i 206.25.63.55 put N5_0_211.ap private

Example for SA-10, WB-10:

tftp -i 206.25.63.55 put N5_0_211.sa private

* NOTE: During the download process you will see the next message:

Code Load Start Session

Erasing Upper Sector

Code Load End Session

wait for crc calculation ...

crc OK ...

Reset To NonActive Code

... please wait .......

8. The unit will auto reset and switch to the non Active code. If the unit succeeds to load the new firmware correctly the unit will auto reset again to make the new code active.

The process is completely automatic and no user intervention is required.

After the last reset is complete make sure you see the new version shows up on the monitor.

BreezeCOM LTD. - Wireless Access Solutions 2000 ©

Our WEB Page:

http://www.breezecom.com/

Technical assistance:

http://www.breezecom.com/ContactUs/brzcnt.htm