Release Note: BreezeNET PRO.11

November 2000

Firmware = 5.0.211

MIB = 3.10

C/S = F  (AP-10, WB-10, SA-10)
           G (SA-40)

#### General

BreezeCOM is pleased to announce the release of new version firmware, Version 5.0.211, for the BreezeNET PRO.11 product family. This release is necessary to support the new flash memory that is being used due to the worldwide shortage of the memory chip that is currently being used. This new firmware can run on previous version of hardware. That is any equipment that has version 5.0.X firmware can be upgraded to Version 5.0.211. 
Please note: AP-10, WB-10, SA-10 with C/S F and SA-40 with C/S G cannot be downgraded with firmware versions prior to 5.0.211

#### New Features

- Add hardware version to the MIB

It is now possible to remotely identify the hardware version, via SNMP. A new instance had been added to the MIB, which will contain the hardware version.

- Enabled configuring of parameter Contention Window
This parameter should be set according to the amount of hidden stations. Hidden stations are most prevalent in access applications, e.g. ISP’s. The greater the number of hidden stations, the larger the size of contention window should be set. This parameter is configurable via the monitor or SNMP.  The possible values are 7, 15, 31, 63. The default value is 7.

#### Bugs Solved

- Solve hidden stations problem 
There were problems when working in hidden stations environment, due to a bug in setting the NAV mechanism.

- Display Rx packets per frequency 
In the monitor, when in site proprietary mode, the display Rx packets per frequency was incorrect. The display has been corrected only in the AP-10 firmware.

- Compatibility problem with AP-10 Ver 4.x 
When a WB-10 Version 5.x.x associated with an AP-10 Version 4.x and there was heavy broadcast traffic occasionally the AP-10 would reset.

- Site proprietary 
In Version 5.0.154 when working in Site Proprietary there was sometimes a reset and a BUS Error message on the monitor.

- SNMP community string displayed
The error message, on the monitor, when an incorrect community string was used, would display in the error message the incorrect community string that was used.  This has been changed, so that the error message does not display the attempted community string.

#### Known Bugs

- In versions 5.0.154 and 5.0.211 when the AP is in Power Save mode the AP occasionally reports a Bus Error and resets. It is recommended that the Power Save be disabled in installations where there is heavy traffic.

Firmware File Names: 

The firmware version is 5.0.211 for the AP10, SA10, SA40, and WB10. This version is delivered as two load files: N5_0_211.AP for the AP10 and N5_0_211.SA for the SA10 SA40 WB10 stations.

Maintaining Present Device Settings After Firmware Upgrade

The new firmware version can be downloaded without losing the existing configuration settings. In order not to lose the existing configuration parameters when you upgrade:

From 5.0.63 and earlier releases of version 5 to 5.0.211 :- The installer must export by TFTP the configuration file. The name of the configuration file is "private.cfg". The get TFTP action causes the configuration parameters to be saved on the FLASH memory. After the upgrade process is completed, the new version: 5.0.211 will restore the saved configuration parameters from the FLASH. 
From version 5.0.143 and higher to 5.0.211:- No need to do anything, the Saving of current configuration is done automatically.

To upgrade to version 5.0.211 , the upgrade package, is obtainable from Technical Support area on the BreezeCOM web site http://www.breezecom.com/TechSupport/techsupp1.html  .

If you have any questions please contact: Technical Support at http://www.breezecom.com/ContactUs/brzcnt.htm Or Product Manager at pm-wlan@breezecom.com